Professional Serial Communication Protocol v1.8 for Digital Loudspeaker Processor

Revision History​

| Version | Changes |  |
| --- | --- | --- |
| v1.0 | Initial release |  |
| v1.1 | - Real-time volume return for +/- commands - Mute functionality added |  |
| v1.2 | Added return values for partial commands |  |
| v1.3 | Modified parameters for return commands |  |
| v1.4 | Comprehensive revisions |  |
| v1.5 | New commands added |  |
| v1.6 | Expanded addressing capabilities |  |
| v1.8 | ​Channel numbering modified​ (Critical Update) |  |

​Baud Rate: 9600 bps, 8 data bits, No parity, 1 stop bit (9600 8N1)

​Channel Mapping Reference​

| ​Logical Channel​ | HEX Value |  |
| --- | --- | --- |
| Input A | 00 |  |
| Input B | 01 |  |
| Input C | 02 |  |
| Input D | 03 |  |
| Output 1 | 04 |  |
| Output 2 | 05 |  |
| Output 3 | 06 |  |
| Output 4 | 07 |  |
| Output 5 | 08 |  |
| Output 6 | 09 |  |
| Output 7 | 0A |  |
| Output 8 | 0B |  |

​Command Specifications​

1. ​Get Channel Mixing Configuration​

- ​Command Format: FE ZZ 11 XX 00 FF (5-byte HEX)

  - ZZ: Device address (01-FE)

  - XX: Channel number (00-0B)

- ​Response: FE ZZ 12 XX YY FF

  - YY: Mixing configuration code:

| HEX | Configuration | ​Components​ |
| --- | --- | --- |
| 00 | Muted | (Not for mute control) |
| 01 | Input A only | A |
| 02 | Input B only | B |
| 03 | Input A + B | A+B |
| 04 | Input C only | C |
| 05 | Input A + C | A+C |
| 06 | Input B + C | B+C |
| 07 | Input A + B + C | A+B+C |
| 08 | Input D only | D |
| 09 | Input A + D | A+D |
| 0A | Input B + D | B+D |
| 0B | Input A + B + D | A+B+D |
| 0C | Input C + D | C+D |
| 0D | Input A + C + D | A+C+D |
| 0E | Input B + C + D | B+C+D |
| 0F | Input A + B + C + D | A+B+C+D |

2. ​Set Channel Mixing Configuration​

- ​Command Format: FE ZZ 10 XX YY FF

- ​Response: FE ZZ 12 XX YY FF (Confirmation)

3. ​Set Channel Gain​

- ​Command Format: FE ZZ 13 XX YY FF

  - YY: Gain value (HEX):

| HEX | Gain (dB) | ​Step​ |
| --- | --- | --- |
| 00 | -40 dB | Min |
| ... | ... | ... |
| 28 | 0 dB | Unity |
| 29 | +1 dB | ... |
| 34 | +12 dB | Max |

- ​Response: FE ZZ 15 XX YY FF

4. ​Get Channel Gain​

- ​Command Format: FE ZZ 14 XX 00 FF

- ​Response: FE ZZ 15 XX YY FF

5. ​Increase Channel Gain (+1 dB)​​

- ​Command Format: FE ZZ 16 XX 00 FF

- ​Response: FE ZZ 15 XX YY FF (Updated gain)

6. ​Decrease Channel Gain (-1 dB)​​

- ​Command Format: FE ZZ 17 XX 00 FF

- ​Response: FE ZZ 15 XX YY FF (Updated gain)

7. ​Mute Control​

- ​Command Format: FE ZZ 18 XX YY FF

  - YY: 00 = Mute, 01 = Unmute

- ​Response: FE ZZ 1A XX YY FF (Confirmation)

8. ​Get Mute Status​

- ​Command Format: FE ZZ 19 XX 00 FF

- ​Response: FE ZZ 1A XX YY FF

9. ​Recall Preset​

- ​Command Format: FE ZZ 20 XX 00 FF

  - XX: Preset number (01-18 HEX = 1-24 decimal)

- ​Response: FE ZZ 21 XX 00 FF

- ​Persistent Effect: Becomes default after reboot

​Critical Implementation Notes​

1. ​Channel Validation​

- Valid XX Range: 00 to 0B (Hex)

- ​Out-of-Range Handling: No response for invalid channels

2. ​Gain Value Mapping​

- ​Full Range: -40 dB (00) to +12 dB (34)

- ​Resolution: 1 dB per HEX increment

3. ​Data Field Conventions​

- YY=00 in requests: Reserved placeholder

- HEX values use uppercase letters (A-F)

4. ​Command Structure​

plaintext

复制

[Start] [Addr] [Cmd] [Ch] [Data] [End]

FE      ZZ     XX    YY    XX     FF

5. ​Response Timeout​

- Default: 100ms

- Retry after 200ms if no response

6. ​Persistent States​

| ​Function​ | ​Survives Reboot​ |
| --- | --- |
| Preset Recall | Yes |
| Mute State | No |
| Gain Settings | Yes |